# CS23532 – COMPUTER NETWORKS LABORATORY

## Practical – 2

### Aim

To study and understand the different types of network cables used in computer networking.

### a) Different Types of Network Cables

The different types of cables commonly used in networking are:

- Unshielded Twisted Pair (UTP) Cable

- Shielded Twisted Pair (STP) Cable

- Shielded and Screened Twisted Pair (SSTP) Cable

- Coaxial Cable

- Fibre Optic Cable

### 1. Unshielded Twisted Pair (UTP) Cable

UTP cable consists of pairs of insulated copper wires twisted together without any external shielding. It is widely used in Ethernet networks.

| Category | Maximum Data Transmission | Advantages / Disadvantages | Application / Use |
| --- | --- | --- | --- |
| Category 3 | Up to 10 Mbps | Advantages: Low cost, easy to install, lightweight and flexible. Disadvantages: More susceptible to electromagnetic interference (EMI) and noise. | 10Base-T Ethernet |
| Category 5 | Up to 100 Mbps | Good performance and relatively inexpensive. | Fast Ethernet |
| Category 5e | Up to 1 Gbps | Improved performance and reduced interference compared with Cat 5. | Fast Ethernet, Gigabit Ethernet |

### 2. Shielded Twisted Pair (STP) Cable

STP cable contains twisted copper wire pairs surrounded by a protective metallic shield. The shielding reduces electromagnetic interference and improves signal quality.

| Category | Maximum Data Transmission | Advantages / Disadvantages | Application / Use |
| --- | --- | --- | --- |
| Category 6 | Up to 10 Gbps | Advantages: Shielded, faster than UTP and less susceptible to noise and interference. Disadvantages: More expensive and requires greater installation effort. | Gigabit Ethernet, 10G Ethernet |
| Category 6a | Up to 10 Gbps | Better shielding and improved performance over longer distances. | Gigabit Ethernet, 10G Ethernet, data centres |

### 3. Shielded and Screened Twisted Pair (SSTP) Cable

SSTP cable provides additional protection against electromagnetic interference by using shielding around the individual pairs and/or the complete cable.

| Category | Maximum Data Transmission | Advantages / Disadvantages | Application / Use |
| --- | --- | --- | --- |
| Category 7 | Up to 10 Gbps | Advantages: Excellent shielding, high performance and low susceptibility to interference. Disadvantages: Expensive and requires careful installation. | Gigabit Ethernet, 10G Ethernet up to approximately 100 m |

### 4. Coaxial Cable

Coaxial cable consists of a central copper conductor, insulation, a metallic shield and an outer protective jacket. It provides good resistance to external interference.

| Type | Maximum Data Transmission | Advantages / Disadvantages | Application / Use |
| --- | --- | --- | --- |
| RG-6 | Up to 10–100 Mbps* | Advantages: High bandwidth, good resistance to interference, low signal loss and versatile. Disadvantages: Bulky and has distance limitations. | Television networks, cable Internet |
| RG-59 | Up to 10–100 Mbps* | Flexible and suitable for short-distance signal transmission. | CCTV, television and video systems |
| RG-11 | Up to 10–100 Mbps* | Suitable for longer cable runs with lower signal loss. | Broadband and television networks |

*Transmission speed depends on the networking technology and equipment used.

### 5. Fibre Optic Cable

Fibre optic cable transmits data as pulses of light through optical fibres. It provides very high bandwidth and can carry data over long distances.

| Type | Maximum Data Transmission | Advantages / Disadvantages | Application / Use |
| --- | --- | --- | --- |
| Single Mode Fibre | Up to 100 Gbps or higher* | Advantages: Very high speed, high bandwidth, long-distance transmission and high security. Disadvantages: Expensive equipment and requires skilled installation. | Long-distance communication, telecom networks, data centres |
| Multi Mode Fibre | Up to 100 Gbps or higher* | High bandwidth, reliable and suitable for shorter distances. | LANs, data centres and campus networks |

*Actual speed and distance depend on the fibre type, transceivers and networking technology. Modern fibre-optic systems can support distances far greater than 100 metres.

### Conclusion

Thus, different types of network cables such as UTP, STP, SSTP, coaxial and fibre optic cables were studied. Each cable has different characteristics, transmission speeds, advantages, disadvantages and applications. UTP is commonly used for LAN connections, coaxial cable is used for television and broadband connections, while fibre optic cable is preferred for high-speed and long-distance communication.